IEI STUDENT RESEARCH PROJECT PROGRAMME (SRPP) 2025–2026

Project Proposal Submission Document

# 1. Title of the Project

Automated Brain Tumour Segmentation from Multi-Modal MRI Using 3D U-Net with a Guarded BraTS Standardisation Pipeline ("BraTS-ifier")

# 2. SRPP Theme / Broad Area

Data Science, Artificial Intelligence, Pattern Recognition and Image Processing, Signal Processing, Biomedical Engineering, Sensors and Measurement Techniques

# 3. Abstract

Manual delineation of brain tumours from magnetic resonance imaging (MRI) is time-consuming, operator-dependent, and a bottleneck in neuro-oncology workflows: a radiologist typically spends 15–45 minutes contouring a single multi-modal study, and resulting contours vary between observers (inter-rater Whole-Tumour Dice ≈ 0.85–0.90). This project presents an end-to-end system that converts a standard hospital brain-MRI study (T1, post-contrast T1ce, T2, FLAIR — in NIfTI or DICOM form) into an automatic tumour segmentation with volumetry and interactive 3D visualisation.

The system has two components: (1) BraTS-ifier, a deterministic, fully guarded standardisation pipeline that converts heterogeneous hospital scans into BraTS-grade format — affine-aware 1 mm isotropic resampling, gradient-based rigid co-registration of four modalities to the T1ce reference, guarded SimpleITK N4 bias-field correction, grow-only placement on the 240×240×155 BraTS canvas, and brain-masked z-score normalisation. Every stage has fail-safe guardrails: if any stage cannot provably improve the data, the original scan is returned untouched with the reason logged. (2) A trained 3D U-Net convolutional neural network (4-channel input, 4-class output, ~23M parameters) trained on BraTS 2021 over 10 epochs (Adam, lr=1e-3, Dice-based loss), reaching Whole-Tumour Dice of 0.8474 on a held-out patient, exported to ONNX Runtime for CPU/GPU-agnostic deployment.

The Gradio application additionally provides a synced multi-planar (MPR) viewer, an in-situ 3D tumour viewer, RANO diameter measurements, STL/GLB mesh and NIfTI-seg exports, a printable HTML report, per-run QA snapshots with JSON provenance, DICOM anonymisation on upload, and an honest partial-scan fallback (anatomy + asymmetry heatmap + PACS re-request checklist) that refuses to fabricate tumour volumes when modalities are missing. The system is research visualisation only — not a certified medical device; all outputs require radiologist verification.

# 4. Introduction and Problem Statement

A standard brain-tumour MRI protocol acquires four co-registered volumes — pre-contrast T1, post-gadolinium T1ce, T2, and FLAIR — from which the radiologist delineates three tumour sub-regions: necrotic/non-enhancing core, peritumoral edema, and enhancing tumour. Manual contouring is slow, suffers inter-rater variability, and does not produce 3D-printable or navigation-ready outputs for surgical planning.

Meanwhile, research segmentation models (notably the BraTS challenge winners) are trained on curated, skull-stripped, co-registered, 1 mm isotropic data — a format raw hospital PACS exports almost never match. The gap between hospital reality and model expectation is the problem this project closes.

Existing solutions face limitations: (1) nnU-Net ensembles achieve ≈0.92 WT Dice but are heavy (ensemble, GPU, Docker-dependent); (2) BraTS-Toolkit preprocessors require Docker and carry AGPL licensing constraints; (3) most tools lack built-in privacy safeguards for patient data.

# 5. Objectives

• Develop a 3D U-Net for automatic segmentation of brain tumours into four classes: Background, Necrotic Core, Peritumoral Edema, and Enhancing Tumour.

• Build BraTS-ifier, a guarded hospital-to-BraTS standardisation pipeline with fail-safe guardrails.

• Export the trained model to ONNX Runtime with verified numerical equivalence (max abs diff < 1e-3 vs PyTorch) and CUDA→CPU fallback.

• Deliver a Gradio-based clinical interface supporting NIfTI and DICOM-ZIP input, MPR/3D visualisation, volumetric reporting (cm³), RANO measurements, STL/GLB/NIfTI-seg exports, HTML reports, QA snapshots, and DICOM anonymisation.

• Implement an honest partial-scan fallback: when fewer than 4 modalities are available, refuse to fabricate tumour volumes; provide asymmetry heatmap and PACS re-request checklist instead.

• Validate with 53 regression tests across 5 test suites, Docker containerisation, and CI pipeline.

# 6. Literature Survey / Background

• Menze, B. et al. "The Multimodal Brain Tumor Image Segmentation Benchmark (BRATS)," IEEE TMI 2015 — establishes the BraTS challenge and multi-modal MRI segmentation task.

• Bakas, S. et al. "BraTS 2021: Identifying the Best Machine Learning Algorithms for Brain Tumor Segmentation" — defines the BraTS 2021 dataset and evaluation protocol used for training.

• Isensee, F. et al. "nnU-Net," Nature Methods 2021 — state-of-the-art self-configuring U-Net; benchmark for accuracy comparison (≈0.92 WT Dice).

• Çiçek, Ö. et al. "3D U-Net," MICCAI 2016 — architectural basis of the segmentation model (4-channel input, 4-class output).

• Tustison, N. et al. "N4ITK: Improved N3 Bias Correction," IEEE TMI 2010 — basis for the guarded N4 bias-field correction stage in BraTS-ifier.

• Wen, P.Y. et al. "RANO criteria," JCO 2010 — basis for RANO-style diameter measurements and response classification (Progressive ≥ +25%, Partial response ≤ −50%, Stable otherwise).

Alternative systems evaluated and not depended upon: BraTS-Toolkit preprocessor (Docker + AGPL licence constraints), BrainLes/ANTs (reference-grade but requires external binaries). These were reviewed and rejected in favour of a pure-Python, dependency-light pipeline.

# 7. Methodology

## 7.1 Data Ingestion and Preprocessing

Input: Four-channel MRI in canonical order T1, T1ce, T2, FLAIR. Accepts (a) individual NIfTI file uploads with explicit or inferred modality-to-file mapping, and (b) a single ZIP archive containing DICOM series folders.

DICOM handling: Recursive DICOM discovery from directory or ZIP; series grouping by SeriesInstanceUID; modality inference from series description, protocol, or image type; scout/localiser series automatically skipped.

Validation: Missing modalities, inconsistent volume shapes, incompatible orientations, corrupt DICOM data, non-uniform slice spacing — all checked with explicit error reporting.

Preprocessing: T2-based brain masking and bounding-box cropping; resampling of each cropped modality to 128×128×128 voxels; per-modality z-score normalization; batch preparation as float32 tensor (1, 4, 128, 128, 128).

## 7.2 BraTS-ifier Standardisation Pipeline

Stage 1 — Affine-aware 1 mm isotropic resampling of each modality.

Stage 2 — Gradient-NCC rigid co-registration of all four modalities to the T1ce reference (chosen because T1↔FLAIR intensities differ but edges match).

Stage 3 — Guarded SimpleITK N4 bias-field correction (skipped if it cannot provably improve the data).

Stage 4 — Grow-only placement on the 240×240×155 BraTS canvas (never crops or distorts anatomy).

Stage 5 — Brain-masked z-score normalisation per modality.

Guardrail principle: every stage returns the original scan untouched with a logged reason if it cannot provably improve data quality — no silent degradation.

## 7.3 Model Architecture and Inference

Architecture: 3D U-Net (Çiçek et al., MICCAI 2016), 4 input channels (T1, T1ce, T2, FLAIR), 4 output classes (Background, Necrotic Core, Peritumoral Edema, Enhancing Tumour), ~23M parameters.

Training: BraTS 2021 dataset, 10 epochs, Adam optimiser (lr=1e-3), Dice-based loss, BraTS label 4 remapped to class 3 during preprocessing.

Performance: Whole-Tumour Dice 0.8474 on held-out patient; Tumour Core and Enhancing Tumour Dice also evaluated; Hausdorff distance reported per class.

Export: PyTorch model exported to ONNX (opset 17) with dynamic axes (batch, depth, height, width); numerical equivalence verified (max abs diff < 1e-3); CUDAExecutionProvider preferred with CPUExecutionProvider fallback.

## 7.4 Post-Processing and Volumetry

Connected-component cleanup per tumour class (min 50 voxels) removes isolated noise while preserving whole-tumour stability.

Physical tumour volume calculation using voxel spacing from the source affine (voxel count × spacing / 1000 = cm³); reported per class and as Total Whole Tumour.

Resampling of predicted class map back into original scan space using nearest-neighbour interpolation.

RANO-style diameters: Dmax and Dperp on axial slices (whole tumour); RANO classification vs prior (Progressive ≥ +25%, Partial response ≤ −50%, Stable otherwise).

## 7.5 Visualisation and Reporting

Multi-planar reconstruction (MPR) viewer with synced axial/sagittal/coronal sliders and tumour overlay.

In-situ 3D tumour surface viewer (marching cubes meshes per class, colour-coded).

Exports: STL meshes per tumour class, GLB combined scene (optional), NIfTI segmentation volume, printable HTML surgical report, per-run QA PNG snapshots, JSON provenance.

## 7.6 Privacy and Honest Fallback

DICOM anonymisation on upload: removes PatientName, PatientID, PatientBirthDate, PatientSex, PatientAge, PatientAddress, InstitutionName, ReferringPhysicianName, PerformingPhysicianName, StudyID, AccessionNumber; hashes PatientID with salt; strips group-0000 command elements.

Partial-scan fallback (Tier 2): when T1ce or FLAIR are missing, the 4-channel U-Net cannot run honestly. Instead: mirror left/right hemispheres to produce an asymmetry heatmap (non-specific, not called a tumour), geometric ellipsoid volume estimate (clearly labeled as estimate), and a PACS re-request checklist (AX T1 pre-contrast, AX T1C post-contrast, AX T2, AX FLAIR as DICOM, one series per modality).

Model health guardrails: .PLACEHOLDER markers flag untrained weights; 🚨 warning banner when placeholder weights are in use; outputs explicitly labeled as research-only, not for diagnosis.

# 8. Expected Outcomes and Deliverables

• Trained 3D U-Net model exported as ONNX file (~23M parameters, opset 17, CUDA/CPU agnostic).

• BraTS-ifier standardisation pipeline with documented guardrails and fail-safe behaviour.

• Gradio web application (http://127.0.0.1:7860) for hospital MRI → tumour segmentation + 3D visualisation.

• Per-case outputs: tumour class volumes (cm³), RANO diameters, MPR snapshots, 3D mesh (STL/GLB), NIfTI segmentation, HTML report, QA PNG, provenance JSON.

• DICOM anonymisation module for privacy-safe hospital data handling.

• 53 regression tests across 5 test suites (ingestion, volumetry, partial-analysis, compare-prior, unified).

• Dockerfile and CI configuration for reproducible deployment.

• Honest partial-scan fallback that never fabricates tumour volumes when modalities are missing.

# 9. Relevance to IEI SRPP Theme

This project directly addresses multiple IEI SRPP 2025–26 indicative themes: (1) Data Science and Artificial Intelligence — 3D U-Net deep learning model for automated image segmentation; (2) Pattern Recognition and Image Processing — multi-modal MRI analysis, co-registration, bias correction, and tumour classification; (3) Biomedical Engineering, Sensors and Measurement Techniques — clinical MRI processing pipeline with volumetry and surgical visualisation outputs; (4) Big Data / IoT / Robotics (health) — structured hospital imaging data pipeline with DICOM/NIfTI ingestion and anonymisation.

Societal relevance: Brain tumour segmentation is a critical step in neuro-oncology treatment planning. Automating and standardising this process reduces radiologist workload, improves contour consistency, and produces 3D-printable / navigation-ready tumour models for surgical planning — directly supporting healthcare improvement, a stated SRPP focus area.

# 10. Work Plan / Timeline

The project is implemented and validated. Remaining SRPP-specific activities align with the programme schedule:

• Proposal submission: February 2026 (per IEI KSC dates: 09th–21st February 2026)

• Expert shortlisting and announcement of selected projects: 5th March 2026

• Midterm evaluation (online mode): 28th March 2026

• Announcement for final presentation (offline at IEI KSC): 10th April 2026

• Synopsis & final project report submission: 24th April 2026

• State-level final presentation: 09th May 2026

Technical milestones already completed: model training on BraTS 2021, ONNX export with equivalence verification, BraTS-ifier pipeline, Gradio application, DICOM anonymisation, partial-scan fallback, 53 regression tests, Docker/CI setup.

# 11. Budget Estimate

The project uses open-source software only; no proprietary licences are required.

• Hardware: GPU workstation for model training (one-time); CPU-only sufficient for inference/deployment.

• Software: Python 3.12, PyTorch, ONNX Runtime, nibabel, pydicom, SimpleITK, GradIO, Plotly, scikit-image, SciPy, NumPy — all open-source (MIT/BSD/Apache).

• Dataset: BraTS 2021 (public, no cost).

• IEI SRPP processing fee: Rs. 1000 (as per programme announcement).

Estimated total project cost (excluding institutional GPU access and processing fee): minimal (open-source stack, public dataset).

# 12. References

1. Menze, B. et al. "The Multimodal Brain Tumor Image Segmentation Benchmark (BRATS)," IEEE Transactions on Medical Imaging, 2015.

2. Bakas, S. et al. "BraTS 2021: Identifying the Best Machine Learning Algorithms for Brain Tumor Segmentation." BraTS 2021 challenge documentation.

3. Isensee, F. et al. "nnU-Net: Self-configuring method for deep learning-based biomedical image segmentation," Nature Methods, 2021.

4. Çiçek, Ö. et al. "3D U-Net: Learning Dense Volumetric Segmentation from Sparse Annotation," MICCAI 2016.

5. Tustison, N. et al. "N4ITK: Improved N3 Bias Correction," IEEE Transactions on Medical Imaging, 2010.

6. Wen, P.Y. et al. "Updated Response Assessment Criteria for High-Grade Glioma: RANO," Journal of Clinical Oncology, 2010.

7. BraTS-Toolkit / BrainLes — evaluated as preprocessing alternatives; not depended upon (Docker + AGPL constraints).

# 13. Keywords

Brain tumour segmentation, BraTS, 3D U-Net, MRI standardisation, N4 bias correction, image registration, ONNX Runtime, volumetry, surgical visualisation, DICOM anonymisation, RANO criteria.

# 14. Technology Stack

Language: Python 3.12.3

Deep Learning: PyTorch ≥2.0.0, ONNX ≥1.14.0, ONNX Runtime ≥1.16.0 (CPU + GPU).

Medical Imaging: nibabel ≥5.1.0, pydicom ≥2.3.0, dicom2nifti ≥2.4.0, SimpleITK ≥2.3.0, scikit-image ≥0.21.0.

Visualisation / UI: GradIO ≥4.0.0, Plotly ≥5.14.0, Matplotlib ≥3.7.0.

Scientific: NumPy ≥1.24.0, SciPy ≥1.10.0.

Testing / Dev: pytest ≥8.0.0, ruff ≥0.4.0, Docker, GitHub Actions CI.

# 15. Scope and Limitations

Scope: Automated brain tumour segmentation from multi-modal MRI with volumetry, 3D visualisation, and reporting — for research and educational use.

Limitations: (1) Research visualisation only — not a certified medical device; all outputs require radiologist verification. (2) Performance degrades on post-op, hemorrhage, pediatric, or low-quality acquisitions. (3) Volume accuracy depends on correct modality identity, registration, orientation, voxel spacing, and acquisition quality. (4) Nearest-neighbour interpolation used when mapping predictions back to original space. (5) DICOM modality identification relies on descriptive metadata tokens. (6) Not a complete training CLI — inference and visualization workflow with pre-trained weights.

# 16. Innovation and Novelty

• Guarded standardisation pipeline (BraTS-ifier) that never silently degrades hospital scan data — every stage has fail-safe guardrails.

• Honest partial-scan fallback that refuses to fabricate tumour volumes when modalities are missing — provides asymmetry heatmap, geometric estimate, and PACS checklist instead.

• Model health guardrails (.PLACEHOLDER markers + 🚨 banner) preventing clinical use of untrained weights.

• Privacy-by-design: DICOM PHI anonymisation on upload, per-run workdir purging, configurable retention.

• End-to-end verified PyTorch↔ONNX numerical equivalence (max abs diff < 1e-3) with CPU/GPU-agnostic deployment.

NOTE: Personal information (student name, college name, guide name, contact details, roll number, etc.) has been excluded as requested. Please fill in these fields directly in the IEI SRPP format_original_backup.pdf form before submission.